| RA No. | MOG-RA-042 |  | Assessment Date: |  | Next Review Date: |  | Overall Risk: | MEDIUM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Location/Vessel: |  | Description of Work: | Working Overboard |
| --- | --- | --- | --- |

2. Individual Tasks/Activities associated with the Work.

| Step No | Brief Description of Task/Activity | Hazard/s for Task/Activity | Who/What is at risk? | Initial Risk (No Controls in Place) |  |  | ##### Description of Control Measures Required | ##### Residual Risk (Controls in Place) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | How Serious? | How Likely? | Overall Risk |  | How Serious? | How Likely? | Overall Risk |

| 1 | Working Overboard | Conflicting work with other vessels or putting personnel at risk to other vessels or operations | W,P | H | P | M | - Inform harbour where required. - Review surroundings and other vessels in vicinity and ensure that work is not undertaken that could pose an increased risk to personnel. | H | U | M |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Fall | W | H | P | M | - Work permit issued - Working Overboard Checklist done - Toolbox talk done - Fall arresters to be used if applicable - Correct PPE worn | H | U | M |
|  |  | Drowning | W | H | P | M | - Wear suitable Life-vest - A Responsible Officer Co-ordinating - Lifebuoy with line at the ready - Only work in daylight | H | U | M |
|  |  | Main Engines being started & clutched-in | W | H | P | M | - Inform Engineers & place sign in E/R control room | H | U | M |
|  |  | Breaking of gantlines of Stage or Bosuns Chair | W | H | P | M | - Check all equipment before use | H | U | M |

| Assessed by: |  | Approved By: |  | Date: |  |
| --- | --- | --- | --- | --- | --- |

Risk Assessment Severity, Likelihood/Probability and Overall Risk Rating

|  |  | ### How Serious (Hazard Severity)? |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Negligible (N) | Slight (S) | Moderate (M) | High (H) | Very High (VH) |
|  | Affects on People > | Negligible injury, no absence from work | Minor injury requiring first aid treatment. | Injury leading to a lost time accident | Involving a single death or serious injury | Multiple Deaths |
|  | Affect on the environment > | Negligible impact to the environment with no clean up required | Minor impact to the environment requiring clean up | Moderate pollution incurring some restitution costs. | Major pollution with short term implications incurring significant restitution costs | Major pollution with long term implication and very high restitution costs |
|  | Affect on Work or Assets > | Negligible loss of work time with no damage to equipment | Minor loss of work time or damage to equipment requiring minor repair | Significant loss of work time or damage to equipment requiring extensive repair | Stoppage of work or damage to equipment requiring major repair | Stoppage of work or extensive damage to equipment requiring replacement |
| How Likely is to Occur? (Likelihood) | Very Unlikely (VU) A freak combination of factors would be required for an occurrence. | LOW | LOW | LOW | LOW | LOW |
|  | Unlikely (U) A rare combination of factors would be required for an occurrence. | LOW | LOW | LOW | MEDIUM | MEDIUM |
|  | Possible (P) Could occur when additional factors are present but unlikely to occur in normal circumstances. | LOW | LOW | MEDIUM | MEDIUM | HIGH |
|  | Likely (L) Not certain to occur but an additional factor will significantly increase the likelihood. | LOW | MEDIUM | MEDIUM | HIGH | HIGH |
|  | Very Likely (VL) An occurrence almost inevitable without controls in place | MEDIUM | MEDIUM | HIGH | HIGH | HIGH |

| LOW RISK | MEDIUM RISK | HIGH RISK |
| --- | --- | --- |
| May be acceptable, however, due care should be employed and task reviewed to see if the risk can be reduced further. | Task should only proceed with authorisation of Master or Manager. Where possible the task should be redefined and/or measures employed to reduce the residual risk. | Task must not proceed. It should be redefined or further control measures put in place to reduce the risk. Controls should be re-assessed prior to the task commencing. |