| RA No. | MOG-RA-059 |  | Assessment Date: |  | Next Review Date: |  | Overall Risk: | LOW |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Location/Vessel: |  | Description of Work: | Retrieving slipped Anchor and chain |
| --- | --- | --- | --- |

2. Individual Tasks/Activities associated with the Work.

| Step No | Brief Description of Task/Activity | Hazard/s for Task/Activity | Who/What is at risk? | Initial Risk (No Controls in Place) |  |  | ##### Description of Control Measures Required | ##### Residual Risk (Controls in Place) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | How Serious? | How Likely? | Overall Risk |  | How Serious? | How Likely? | Overall Risk |

| 1.1 | Working in a Hazardous surrounding with snap back zones etc. | Risk of Messenger line parting, block failing etc | W,A | M | P | M | - Check all the items used for work. - Ensure no prior damage to the same. - Ensure all the blocks are secured at strong points that can withstand the weight applied. | M | U | L |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.2 | Windlass Operation. | Explosion of Windlass / Hydraulic Pipes. | W,P,A,E | H | VU | L | - Correct PPE Worn. - Regular Routine Maintenance & Surveys / Inspections. - Correct Training. | H | VU | L |
| 1.3 | Anchoring. | Slipping / Tripping on Deck | W | M | U | L | - Correct PPE. - Area to be Kept Clean & Tidy. | M | U | L |
| 1.4 | Anchoring. | Muscular Strain Whilst Operating Windlass | W | S | U | L | - Correct Training - Minimum Manning Level for Operation Adhered Too. | S | VU | L |
| 1.5 | Anchor chain breaking/ parting | Severe injury to crew | W | S | L | M | - Anchor chain should be inspected on the AH Tug. If signs of damage found, operation will be aborted. | S | VU | L |

| Assessed by: |  | Approved By: |  | Date: |  |
| --- | --- | --- | --- | --- | --- |

Risk Assessment Severity, Likelihood/Probability and Overall Risk Rating

|  |  | ### How Serious (Hazard Severity)? |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Negligible (N) | Slight (S) | Moderate (M) | High (H) | Very High (VH) |
|  | Affects on People > | Negligible injury, no absence from work | Minor injury requiring first aid treatment. | Injury leading to a lost time accident | Involving a single death or serious injury | Multiple Deaths |
|  | Affect on the environment > | Negligible impact to the environment with no clean up required | Minor impact to the environment requiring clean up | Moderate pollution incurring some restitution costs. | Major pollution with short term implications incurring significant restitution costs | Major pollution with long term implication and very high restitution costs |
|  | Affect on Work or Assets > | Negligible loss of work time with no damage to equipment | Minor loss of work time or damage to equipment requiring minor repair | Significant loss of work time or damage to equipment requiring extensive repair | Stoppage of work or damage to equipment requiring major repair | Stoppage of work or extensive damage to equipment requiring replacement |
| How Likely is to Occur? (Likelihood) | Very Unlikely (VU) A freak combination of factors would be required for an occurrence. | LOW | LOW | LOW | LOW | LOW |
|  | Unlikely (U) A rare combination of factors would be required for an occurrence. | LOW | LOW | LOW | MEDIUM | MEDIUM |
|  | Possible (P) Could occur when additional factors are present but unlikely to occur in normal circumstances. | LOW | LOW | MEDIUM | MEDIUM | HIGH |
|  | Likely (L) Not certain to occur but an additional factor will significantly increase the likelihood. | LOW | MEDIUM | MEDIUM | HIGH | HIGH |
|  | Very Likely (VL) An occurrence almost inevitable without controls in place | MEDIUM | MEDIUM | HIGH | HIGH | HIGH |

| LOW RISK | MEDIUM RISK | HIGH RISK |
| --- | --- | --- |
| May be acceptable, however, due care should be employed and task reviewed to see if the risk can be reduced further. | Task should only proceed with authorisation of Master or Manager. Where possible the task should be redefined and/or measures employed to reduce the residual risk. | Task must not proceed. It should be redefined or further control measures put in place to reduce the risk. Controls should be re-assessed prior to the task commencing. |